# IPCTECH Motherboard User's Manual

QY-MB-H81-ATX

1.Models and Attentions

ATX Mother Board

- 1.1 Models

This manual is applied to following models:

| Chipset | COM | LAN | USB | PCIE | PCI | DVI-D | VGA | Mini-PCIE | SATA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H81 | 6 | 2 | 10 | 1*16X 1*4X 1*1X | 4 | 1 | 1 | 1 | 3 |

## 1.2 Attentions

1. Notes under a table or figure indicate the difference of models, or alternative definition of specific pin of the header (jumper/connector).

2. How to identify the first pin of a header or jumper

Usually, there is a thick line or a triangle near the header’s or jumper’s pin 1.

Square pad, which you can find on the back of the motherboard, is usually used for pin 1.

ATX Mother Board

- 2.Specification

| - Model | - QY-MB-H81-ATX |
| --- | --- |
| - CPU | - Support Intel Pentium / Celeron / 4th Generation Core i3/i5/i7 CPU, LGA 1150 |
| - Chipset | - Intel H81 [1], TDP 4.1W |
| - Dispaly | - 1*VGA - 1*DVI-I Port (Only support DVI-D Signal) |
| - Memory | - Support DDR3-1333/1600 MHz, 2*DIMM Slot, up to 16GB 【2】 |
| - Storage | - 1*SATA 3.0+2*SATA 2.0 |
| - Ethernet | - 2*Intel I210 GBE LAN Chip (10 / 100 / 1000 Mbps,RJ-45) |
| - Audio | - Reltek ALC897 5.1 Channel HDA Codec, - Support MIC / Line-out Ports |
| - Expansion Slots | - 1*PCIE 16X Slot - 1*PCIE 4X Slot [4] - 1*PCIE 1X Slot - 4*PCI Slot - 1*Mini-PCIE Slot (WIFI+4G, with 1*Full-Size SIM Card Slot |
| - COM | - 5*RS-232+1*RS-232 / 485 [3] |
| - USB | - 2*USB 3.0 (Type-A, Rear I/O) - 2*USB 2.0 (Type-A, Rear I/O) - 6*USB 2.0 (Header, Internal) |

ATX Mother Board

| - Other Ports | - 8 * GPIO - 1 * Audio Connector + 1 * Front Audio - 1 * Mini PCI-E [5] - 1 * LPT Header - 1 * SIM Connector - 1 * PS/2 Connector + 1 * PS/2 Pin Header [6] - 2 * ATX Power Input Connector |
| --- | --- |
| - System | - Windwos 7, Linux |
| - Temperature | Storage: -20~75℃ Operating: 0~60℃ |
| - BIOS | AMI UEFI BIOS (Support Watchdog Timer) |
| - Size | 305mm * 220mm |

Notes:

Chipset can be customized into Intel B85.

[2]: Due to the restriction of Windows 32bit OS, when applied more than 4G memory, 32bit OS may

detect less than actual size.

[3]: Signal of J_COM3 is RS232 by default. It also can be RS485 via Jumper JCOM3_485.

[4]: This PCI-E X4 slot can only support PCI-E X1 device for it just have PCI-E X1 signal.

[5]: Mini PCI-E1 slot supports mSATA (SATA 3.0) by default at the cost of invalidating SATA1.It can also support 3G/WIFI if specialized. When it supports 3G, one of the USB ports of F_USB3 will be invalid.

[6]: The PS/2 Connector and the PS/2 Pin Header share the same signal; they can’t be accessed

simultaneously.

ATX Mother Board

- 3.Functional Block Diagram

ATX Mother Board

4. Jumpers / Headers and Connectors

ATX Mother Board

| Jumpers / Headers and Connectors |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Front USB Pin Header | P8 | 19 | DIMM Slots |  |
| 2 | Parallel Pin Header | P8 | 20 | CPU Slot |  |
| 3 | RS485/232 Select Jumper | P9 | 21 | ATX1 Power Input Connector |  |
| 4 | COM3-6 Control Jumpers | P9 | 22 | Mini-PCIE Slot |  |
| 5 | GPIO Pin Header | P10 | 23 | PCIE X 16 Slot |  |
| 6 | COM3-6 Pin Header | P10 | 24 | DIMM Slots |  |
| 7 | Front Panel Pin Header | P11 | 25 | CPU Slot |  |
| 8 | Debug Pin Header |  | 26 | PCI Slots |  |
| 9 | COMS Clear Jumper | P11 | 27 | PS/2 Connector |  |
| 10 | Debug Jumper |  | 28 | VGA Connector |  |
| 11 | System Fan Connectors | P11 | 29 | COM2 Connector |  |
| 12 | CPU Fan Connector Front | P12 | 30 | USB 2.0 Connectors |  |
| 13 | Keyboard and Mouse Pin Header | P12 | 31 | USB 3.0 Connectors |  |
| 14 | COM1-2 Control Jumpers | P12 | 32 | Audio Connector |  |
| 15 | Audio Pin Header | P12 | 33 | DVI Connector |  |
| 16 | SATA3.0 Connector |  | 34 | COM1 Connector |  |
| 17 | SATA2.0 Connector |  | 35 | LAN Connector |  |
| 18 | ATX2 Power Input Connector |  | 36 | LAN Connector |  |

ATX Mother Board

5.Definition of Jumpers / Headers and Connectors

- 1)Front USB Pin Headers (5*2 Pin 2.54mm)

| - Location | - Header | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | - F-USB1 | - 1 | + 5V | - 2 | +5V |
|  |  | - 3 | USB1- | - 4 | USB2- |
|  |  | - 5 | - USB1+ | - 6 | USB2+ |
|  |  | - 7 | GND | - 8 | GND |
|  |  |  |  | - 10 | NC |
|  | - F-USB2 | - 1 | + 5V | - 2 | +5V |
|  |  | - 3 | USB3- | - 4 | USB4- |
|  |  | - 5 | - USB3+ | - 6 | USB4+ |
|  |  | - 7 | GND | - 8 | GND |
|  |  |  |  | - 10 | NC |
|  | - F-USB3 | - 1 | + 5V | - 2 | +5V |
|  |  | - 3 | USB7- [1] | - 4 | USB6- |
|  |  | - 5 | - USB5+ [1] | - 6 | USB6+ |
|  |  | - 7 | GND | - 8 | GND |
|  |  |  |  | - 10 | NC |
|  | - F-USB4 | - 1 | + 5V | - 2 | +5V |
|  |  | - 3 | USB7- [2] | - 4 | USB8- [2] |
|  |  | - 5 | - USB7+ [2] | - 6 | USB8+ [2] |
|  |  | - 7 | GND | - 8 | GND |
|  |  |  |  | - 10 | NC |

- Noties:

[1]: When Mini-PCIE Slot supports 3G devices, one of the USB ports of F-USB3 will be invalid.

[2]: F-USB4 can be accessed when mounting B85 chipset.

ATX Mother Board

- 2)Parallel Pin Header (13*2 Pin 2.54mm)

| - Location | - Header | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | - LPT1 | - 1 | STROBE# | - 2 | AUTOFEED# |
|  |  | - 3 | DATA0 | - 4 | ERROR# |
|  |  | - 5 | DATA1 | - 6 | INIT# |
|  |  | - 7 | DATA2 | - 8 | SLCTIN# |
|  |  | - 9 | DATA3 | - 10 | GND |
|  |  | - 11 | DATA4 | - 12 | GND |
|  |  | - 13 | DATA5 | - 14 | GND |
|  |  | - 15 | DATA6 | - 16 | GND |
|  |  | - 17 | DATA7 | - 18 | GND |
|  |  | - 19 | ACKNLG# | - 20 | GND |
|  |  | - 21 | BUSY | - 22 | GND |
|  |  | - 23 | PE | - 24 | GND |
|  |  | - 25 | SLCT | - 26 | N/C |

- 3)RS485/RS232 Select Jumper (3*2 Pin 2.54mm)

| - Location | - Header | - Settings | - Function |
| --- | --- | --- | --- |
|  | - JCOM3-485 | 1-3、2-4(Default) | RS232 |
|  |  | 3-5、4-6 | RS485 |

ATX Mother Board

- 4)COM3-6 Control Jumper (6*2 Pin 2.54mm)

| - Location | - Jumper | - COM | - Setting | - Function |
| --- | --- | --- | --- | --- |
|  | JCOM3-6-P1 | COM3 | 1-3 [1] | Pin1: DCD |
|  |  |  | 3-5 | Pin1: + 5V |
|  |  | COM4 | 2-4 (Default) | Pin1: DCD |
|  |  |  | 4-6 | Pin1: + 5V |
|  |  | COM5 | 7-9 (Default) | Pin1: DCD |
|  |  |  | 9-11 | Pin1: + 5V |
|  |  | COM6 | 8-10 (Default) | Pin1: DCD |
|  |  |  | 10-12 | Pin1: + 5V |

- Noties:

- [1]: Jumper 1-3 isn’t on by default.

- 5)GPIO Header （6*2 Pin 2.00mm)

| - Location | - Jumper | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | J-GPIO1 | 1 | PCH-GPO68 （0x1C48, bit 4） | 2 | PCH-GPI69 （0x1C48, bit 5） |
|  |  | 3 | PCH-GPO70 （0x1C48, bit 6） | 4 | PCH-GPI71 （0x1C48, bit 7） |
|  |  | 5 | GND | 6 | PCH-GPI1 （0x1C0C, bit 1） |
|  |  | 7 | PCH-GPO6 （0x1C0C, bit 6） | 8 | PCH-GPI7 （0x1C0C, bit 7） |
|  |  | 9 | PCH-GPO17 （0x1C0E,bit 1） | 10 | + 5V [1] |
|  |  |  |  | 12 | NC |

Noties: All GPIOs can be programmed to input or output.

[1]: GPIO output is 5V signaling by default, 3.3V is available if specified (resistor selectable).

ATX Mother Board

- 6)COM3-6 Pin Headers (5*2 Pin 2.54mm)

| - Location | - Jumper | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | J-COM3 | 1 | RS232: DCD3 [1] RS485: DATA- | 2 | RS232: DSR3 RS485: N/C |
|  |  | 3 | RS232: RXD3 RS485: DATA+ | 4 | RS232: RTS3 RS485: N/C |
|  |  | 5 | RS232: TXD3 RS485: N/C | 6 | RS232: CTS3 RS485: N/C |
|  |  | 7 | RS232: DTR3 RS485: N/C | 8 | RS232: RI3 RS485: N/C |
|  |  | 9 | GND |  |  |
|  | J-COM4 | 1 | DCD4* | 2 | DSR4 |
|  |  | 3 | RXD4 | 4 | RTS4 |
|  |  | 5 | TXD4 | 6 | CTS4 |
|  |  | 7 | DTR4 | 8 | RI4 |
|  |  | 9 | GND |  |  |
|  | J-COM5 | 1 | DCD5* | 2 | DSR5 |
|  |  | 3 | RXD5 | 4 | RTS5 |
|  |  | 5 | TXD5 | 6 | CTS5 |
|  |  | 7 | DTR5 | 8 | RI5 |
|  |  | 9 | GND |  |  |
|  | J-COM6 | 1 | DCD6* | 2 | DSR6 |
|  |  | 3 | RXD6 | 4 | RTS6 |
|  |  | 5 | TXD6 | 6 | CTS6 |
|  |  | 7 | DTR6 | 8 | RI6 |
|  |  | 9 | GND |  |  |

- Noties:

- * These signals are depending on relevant Jumpers (e.g. Pin1 of J_COM4 depends on

- JCOM3-6_P1 Jumper), to find more details, check the above table (Location 4).

ATX Mother Board

- [1]: This signal is RS232 by default. It also can be other signal via JCOM3-6_P1 and

- JCOM3_485(Location 4 and Location 3).

- 7)Front Panel Pin Header (5*2 Pin 2.54mm)

| - Location | - Jumper | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | F-PANEL1 | 1 | HD LED+ | 2 | Power LED+ |
|  |  | 3 | HD LED- | 4 | Power LED- |
|  |  | 5 | RESET- | 6 | PWR+ |
|  |  | 7 | RESET+ | 8 | PWR - |
|  |  | 9 | N/C | 10 |  |

- 9)COMS Clear Jumper (3*1 Pin 2.54mm）

| - Location | - Header | - Settings | - Function |
| --- | --- | --- | --- |
|  | - JCOM3-485 | 1-3、2-4(Default) | RS232 |
|  |  | 3-5、4-6 | RS485 |

11）System and Auxiliary Fan Connectors (4*1 Pin 2.54mm)

| - Location | - Connector | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | SYS-FAN1 | 1 | GND | 2 | +12V |
|  |  | 3 | FAN Speed Detection |  |  |
|  | SYS-FAN2 | 1 | GND | 2 | +12V |
|  |  | 3 | FAN Speed Detection |  |  |

ATX Mother Board

- CPU Fan Connector (4*1 Pin 2.54mm)

| - Location | - Connector | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | CPU-FAN1 | 1 | GND | 2 | +12V |
|  |  | 3 | FAN Speed Detection | 4 | FAN Speed Control |

- Keyboard and Mouse Pin Header (6*1 Pin 2.00mm)

| - Location | - Header | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | J-KBMS1 | 1 | KB-CLK | 2 | KB-DATA |
|  |  | 3 | MS-CLK | 4 | GND |
|  |  | 5 | + 5V | 6 | MS-DATA |

- COM1-2 Control Jumpers (3*2 Pin 2.00mm)

| - Location | - Jumper | - Settings | - Function |
| --- | --- | --- | --- |
|  | - JCOM1-2-P1 | 1-3、2-4(Default) | Pin1: DCD |
|  |  | 3-5、4-6 | Pin1: + 5V |

- Front Audio Pin Header (5*2 Pin 2.54mm)

| - Location | - Header | - Pin | - Definition | - Pin | - Definition |
| --- | --- | --- | --- | --- | --- |
|  | F-AUDIO1 | 1 | FP-MIC-L | 2 | GND |
|  |  | 3 | FP-MIC-R | 4 | + 3.3V |
|  |  | 5 | FP-OUT-R | 6 | SENSE1_RETURN |
|  |  | 7 | GND | 8 |  |
|  |  | 9 | FP-OUT-L | 10 | SENSE2-RETURN |

[END]